[image: image1.png]WIKICKER.

$15L7, S12L7, S10L7, S8L7
SOLO() BARICG





Congratulations! You have just purchased one of the most advanced subwoofers in the history of car audio. Your KICKER Solo-Baric is the latest in kicker's trend-setting Solo-Baric line, designed and engineered to deliver the ultimate in low bass performance and unmatched durability when installed in a properly designed system. Thanks for buying KICKER. Enjoy.

Solo-Baric Features

Cast Aluminum Basket (patent pending) for strength and to reject stray

magnetic fields

Perimeter Venting in basket for continuous voice coil cooling and highly linear response

Highly Rigid Cone (patent pending) with high internal damping and excellent

resistance to sunlight, pollution, and moisture

Inverted Structural Dome™ further increases cone's resistance to flex under pressure

Stitched Surround (patent pending) prevents cone/surround separation at

high excursions

Blackened Motor Parts for superior heat dissipation and long life Dual Ultra-Length Voice Coils for high excursion and flexible wiring options

High-Temp Kapton Voice Coil Former prevents cone meltdown to minimize warping and rubbing

One piece UnlPlate™ pole piece and back plate:

Blackened Pole Piece provides effective heat transfer and cool operation for maximum power handling

One-Plece Hyper-Extended Pole reduces non-linear magnetic fields around the voice coil gap for even more heat sinking and excellent cone motion control Vented Pole Piece releases pressure under dustcap for freer cone motion and enhanced low bass

Sleeved High-Power Tinsel Lead Wires to resist lead breakage and power-robbing resistance losses

Spring-Loaded Nickel Plated Terminals for solid connections with large gauge speaker wire

Rubatex front and rear gaskets for airtight seal

AMERICA 'S MUSIC MACHINES
RECOMMENDED ENCLOSURES

Your Solo-Baric subwoofers are designed for massive sub-bass performance when installed in a simple sealed enclosure. Sealed bows are known for extended low frequency capability with a smooth gradual roll off charactenscic - elaborate hard-to-build boxes aren't needed to get the best bass response.

Because of the Solo-Bane's high moving mass, it is essential that the enclosure size recommendations be followed exactly. Failure to use the correct box size can result in poor sound quality or premature woofer failure DO NOT USE SOLO-BARIC WOOFERS IN THE FOLLOWING ENCLOSURE DESIGNS:
Vented or ported enclosures

Freeair or infinite baffle mounting

Double vented "7th Order" bandpass enclosure

Passive radiator designs

The cardboard support inside your Solo-Baric's shipping carton can be used as a template for cutting the speaker mounting hole in the enclosure you build. Since the cardboard cut-out is the same size as the basket of the L7, when you make the cut along the line drawn through the template, make sure to cut right on the line to get the correct opening size.

These enclosure volume recommendations are for the airspace inside the enclosure and include the displacement of the woofer. Do not make the airspace any greater than recommended.

Enclosure Volume Recommendations
Model
Minimum Enclosure Volume
Maximum Enclosure Volume

S8L7
.53 cubic feet (9.5 L)
.4 cubic feet (11.5 L)

S10L7
.66 cubic feet (18.71)
8 cubic feet (22.7 L)

S12L7
.88 cubic feet (24.9 L)
1.1 cubic feet (51.2 L)

S15L7
1.5 cubic feet (42.5 L|
1.8 cubic feet (51.0 L)

A Note on Power Handling Capability
Solo-Baric subwoofers will handle their full rated power In any of the recommended enclosures, minimum or maximum. The smaller enclosures are best for use in limited space applications. The larger (maximum) recommended enclosures will yield slightly more bass at the lowest frequencies. The power ratings given are assuming that both voice coils are used. You should never hook up a dual voice coil woofer using only one coil-

2
The following shows examples of the minimum enclosure sizes for each of the Solo-Baric L7s. Panel dimensions assume 3/4" wood thickness.

[image: image2.png]&)





Panel dimensions for minimum recommended enclosures:

Model
Volume (cu.ft.)
Panel A
Panel В
Panel С

S8L7
0.33
10-хЮ"
10"х8-
8"Х8.5"

S10L7
0.66
12"х12"
12"х10.5"
10.5-Х10.5"

S12L7
0.88
13"х13"
13"х11.5"
11.5"х11.5"

S15L7
1.5
16.5"х16.5"
16.5"x11.5"
15-Х11.5"

Soto-Baric cutout specifications:

Corner Radius for S15L7 - 1.0" Comer Radius for S12L7, S10L7. S8L7 - .625"

[image: image3.png]



3
BUILDING YOUR ENCLOSURE

The actual construction of the enclosure is fairly simple, but some rules must be fol​lowed to make sure your cabinet is solid and airtight:

Use very sturdy wood, such as 3/4" MDF (medium density fiberboard) or multi-ply marine plywood. Avoid low density material such as common particle board and low grades of ply​wood.

Join the edges with a high quality wood glue. Do not use silicone, hot glue or caulking materials to connect two pieces of wood.

Use drywall screws or a pneumatic staple gun to hold it together while the glue dries. A screw or staple every 3" will suffice.

Use of triangle shaped braces is recommended to reduce panel vibration.

[image: image4.png]



x-3"forS15L7,S12L7 x-2-for S10L7, S8L7

When using more than one subwoofer, each one should have it's own separate air space. A terminal cup is recommended for making the wiring connection to your enclosure. If necessary, a hole slightly larger than the wire used, sealed with RTV silicone, is acceptable.

Don't move the wire until the silicone is completely dry.

Once the box is built, use RTV silicone to seal all the inside joints. The enclosure must be air tight.

Do not use RTV silicone around the woofer opening Silicone on the basket will void your warranty! If necessary, use rope caulk or plumber's putty.

Add enough polyester fiber material (commonly used as pillow stuffing) to fill the box half way This will help smooth out the response of your woofer.
4
WIRING
sg-o Bar'c 17 woofers are available with dual 2 Ohm and dual 4 Ohm voice coils. Both coils should always be hooked up.
The dual 2 Ohm woofer will give you a 1 Ohm or a 4 Ohm load depending on if the coils are wired in parallel or in series. The dual 4 Ohm woofer will give a 2 Ohm or 8 Ohm load depending on how they are wired.

The terminals with solid red and black markings are for one voice coil. The terminals with the white dots are for the second voice coil.

Parallel Wiring
[image: image5.png]



Dual 2 Ohm - 1 Ohm load  Dual 4 Ohm - 2 Ohm load
Series Wiring
[image: image6.png]



5
Dual 2 Ohm = 4 Ohm load Dual 4 Ohm  8 Ohm load

SPECIFICATIONS

Model
Max amp power (watts)
Sensitivity (dB iw/lm)
Voice Coil (in.)
fs (Hz)
Vas

(cu.ft)
Qts
Xmax (in.)
Mounting Cutout (in.)
Outer flange (in.)
Mounting depth (in.)

S15L7
1000
90.2
2.5
23.0
9.9
.477
.60
14.1 х 14.1
15.6х15.6
7.83

S12L7
750
89.1
2.0
29.7
31
.419
.50
11.2 х 11.2
12.54 х 12.54
7.00

S10L7
600
87.3
2.0
35.1
1.2
.410
.50
9.5х9.5
10.66 х 10.66
6.25

S8L7
450
84.5
2.0
43.5
.46
.551
.41
7.6 х 7.6
8.8х8.8
463

Based on measurements of Dual 2 Ohm models

SPEAKER SYSTEMS LIMITED WARRANTY
Stillwdter Designs warrants this product to be free from defects in material and workmanship under normal use for a period of one (1) year from date of original purchase, unless this product is labeled "B Stock", in which case it is warranted for ninety (90) days from date of purchase. Should service be neces​sary under this warranty for any reason due to manufacturing defect or malfunction during the warranty period, Stillwater Designs will replace or repair (at its discretion) the defective merchandise with equiva​lent merchandise at no charge. Warranty replacements on "B-Stock" may have cosmetic scratches and blemishes. Discontinued products may be replaced with equivalent products.

This warranty Is valid only for the original purchaser and is not extended to owners of the product subsequent to the original purchaser. Any applicable implied warranties are limited in duration to a peri​od of the express warranty as provided herein beginning with the date of the original purchase at retail. and no warranties, whether express or implied, shall apply to this product thereafter. Some states do not allow limitations on implied warranties, therefore these exclusions may not apply to you. '- ' This warranty gives you specific legal rights; however you may have other rights that vary from state to state.

WHAT TO DO IF YOU NEED WARRANTY OR SERVICE
Defective merchandise must be returned to your local Authorized Stillwater Designs (Kicker/Impulse) Dealer for warranty. Assistance in locating an Authorized Dealer can be obtained by writing or calling Stillwater Designs direct. You can confirm that a dealer is authorized by asking to see a current authorized dealer window decal.                                    ""

If it becomes necessary for you to return defective merchandise, call the Kicker Customer Service Department at (405)624-8510 for a Return Authorization (RA) number. Package all defective items in the original container or in a package that will prevent shipping damage, and return to

Stillwater Designs, 5021 North Perkins Road, Stillwater, OK 74075
The RA number must be clearly marked on the outside of the package. Return only defective compo​nents. Return of entire cabinets, system packs, pairs, etc. increases your return freight charges. Non-defective items received will be returned freight collect.

Include a dated prool-of-purchase from an Authorized Dealer. Warranty expiration on items returned without proof-of-purchase will be determined from the manufacturing date code. Coverage may be invalidated if this date is greater than one (1) year previous to the date item is sent in. Freight must be prepaid; items received freight collect will be refused.

Failure to follow these steps may void your warranty. Any questions can be directed to the Kicker Customer Service Department at (405)624-8510.

WHAT IS NOT COVERED?
This warranty is valid only if the product is used for the purpose for which it was designed. It does not cover:

Install slips (screwdriver holes)

Damage caused by exposure to water

and/or excessive heat.

Damage through negligence, misuse, or

accident.

Items physically damaged due to abuse.

Freight damage.

The cost of shipping product to Stillwater

Designs Service.


Items previously repaired by any

unauthorized repair facility.

Items returned from unauthorized

individuals or dealers.

Return shipping on non-defective items.

Speakers damaged due to amplifier

clipping or distortion.

Speakers with silicon caulk used for gasket

material.

HOW LONG WILL IT TAKE?
Stillwater Designs maintains a goal of 24-hour service for all returns. Delays may be incurred if lack of replacement inventory or parts is encountered.

INTERNATIONAL WARRANTY
Contact your International Stillwater Designs dealer or distributor concerning specific procedures for your country's warranty policies,                              cvr-ii i и/Атт
^_^и_^^^——^__^^в^,^^^^^^^^^^^__1н^1^и——^^-^^^^>^^———^^^^^_^^^^^_^———^>^в^и^^^^^^_——^^^^^——^—^^^^^^——ч—тчМ^9^^^~~~
P.O. Box 459 • Stillwater, Oklahoma 74076 • U.S.A. • 405 624-8510   V^7 '   »
[image: image7.png]



KICKER drivers are capable of producing sound levels that can permanently damage your hearing! Turning up a system to a level that has audible distortion is more damaging to your ears than listening to an undistorted system at the same volume level. The threshold of pain is always an Indicator that the sound level Is too loud and may permanently damage your hearing. Please use common sense when controlling volume!

